Verification of an AMBA APB Slave using UVM

A UVM-Based Functional Verification Project

Ahmed Razi Ullah

BEE 13 B

366191

# 1. Introduction

This project develops a UVM-based verification environment for an AMBA APB slave containing a 32-word by 32-bit memory. The environment drives constrained-random write data to all addresses, reads the data back, and uses a self-checking scoreboard to confirm data integrity. The work covers APB protocol timing, UVM component design, monitor-to-scoreboard transaction flow, and waveform-level debugging.

## 1.1 Abstract

Modern SoCs contain many peripherals connected over standard buses. Verifying these interfaces is a major part of a chip's development effort. This project was chosen to build hands-on experience with the UVM methodology on a real, industry-standard protocol.

## 1.2 Objectives

- Understand APB transfer timing (setup and access phases).

- Build a reusable UVM testbench with a driver, monitor, scoreboard and agent.

- Automatically check that data written to the slave is returned correctly on reads.

- Inspect waveforms to confirm protocol-correct behavior.

## 1.3 Scope

The verified DUT is a single APB slave with a 32-entry memory. Error responses (PSLVERR), byte strobes (PSTRB) and functional coverage are outside the current scope and are listed as future work (Section 10).

# 2. Background: The APB Protocol

APB (Advanced Peripheral Bus) is a low-bandwidth, low-power bus in ARM's AMBA family, intended for peripheral registers.

## 2.1 Signals

| Signal | Direction (master view) | Purpose |
| --- | --- | --- |
| PCLK | Input | Bus clock; all transfers are sampled on the rising edge |
| PRESET | Input | Reset (active-high in this implementation) |
| PADDR[31:0] | Output | Target address |
| PSEL | Output | Selects the slave |
| PENABLE | Output | Marks the access phase |
| PWRITE | Output | 1 = write, 0 = read |
| PWDATA[31:0] | Output | Write data |
| PRDATA[31:0] | Input | Read data |
| PREADY | Input | Slave indicates the transfer can complete |

The interface file also declares PSTRB and PSLVERR; these are not implemented in the DUT (see Section 9).

## 2.2 Transfer Phases

- Idle: PSEL = 0, PENABLE = 0.

- Setup: PSEL = 1, PENABLE = 0 for one clock; address, direction and write data are presented.

- Access: PSEL = 1, PENABLE = 1; the transfer completes when PREADY = 1.

# 3. Design Under Test

Module AMBA_APB (design.sv) is a slave with:

- A 32 x 32-bit memory array.

- A three-state FSM: IDLE, SETUP, ACCESS.

- On the access phase, a write stores PWDATA at PADDR; a read places mem[PADDR] on PRDATA; PREADY is asserted.

## 3.1 State Transitions

| Current State | Condition | Next State |
| --- | --- | --- |
| IDLE | PSEL=1, PENABLE=0 | SETUP |
| IDLE | PSEL=0, PENABLE=0 | IDLE |
| SETUP | PSEL=1, PENABLE=1 | ACCESS (transfer performed, PREADY=1) |
| SETUP | PSEL=0, PENABLE=0 | IDLE |
| ACCESS | PSEL=1, PENABLE=1 | ACCESS |
| ACCESS | PSEL=0, PENABLE=0 | IDLE |

# 4. Verification Plan

## 4.1 Features to Verify

| ID | Feature | Method | Pass Criterion |
| --- | --- | --- | --- |
| F1 | Write to every address 0–31 | Sequence writes random data | Monitor observes 32 writes |
| F2 | Read back every address 0–31 | Sequence reads all addresses | Monitor observes 32 reads |
| F3 | Data integrity | Scoreboard compares read vs written | All comparisons match |
| F4 | Protocol timing | Waveform inspection | PSEL precedes PENABLE; PREADY asserted in access phase |
| F5 | Slave responsiveness | Monitor watchdog | PREADY high within 10 cycles; no UVM_ERROR |

## 4.2 Pass / Fail Criteria

The test passes when the scoreboard reports zero failed comparisons and the UVM report shows UVM_ERROR : 0 and UVM_FATAL : 0.

# 5. Testbench Architecture

Data flow: sequence → sequencer → driver → pins → DUT, and pins → monitor → scoreboard.

## 5.1 Components

| Component | Responsibility |
| --- | --- |
| apb_top | Generates a 10 ns-period clock, applies reset for 15 ns, instantiates DUT and interface, stores the virtual interface in uvm_config_db, calls run_test("apb_base_test") |
| apb_base_test | Builds the environment, raises an objection, starts the sequence on the agent's sequencer |
| apb_env | Instantiates agent and scoreboard; connects monitor analysis ports to the scoreboard |
| agent | Contains sequencer, driver and monitor |
| first_sequence | Generates 32 write items followed by 32 read items |
| tx_item | Sequence item with a constraint keeping PADDR in 0–31 |
| driver | Converts items to pin-level activity |
| monitor | Two concurrent processes (write and read) that reconstruct transactions |
| scoreboard | Receives write and read transactions through uvm_analysis_imp ports and compares them |

# 6. Implementation Details

## 6.1 Stimulus

The sequence writes $urandom data to addresses 0 to 31 in order, then reads addresses 0 to 31 in order. Burst lengths are set by WRITE_BURST_LENGTH and READ_BURST_LENGTH (both 32).

## 6.2 Driver

After reset is released, the driver waits for a rising clock edge, gets an item and drives:

- Setup: PWRITE, PSEL, PADDR (and PWDATA for writes).

- Access: one clock later, PENABLE is asserted.

- It waits until PREADY is high, then clears PENABLE and PSEL and calls item_done().

## 6.3 Monitor

The monitor runs write and read collection in parallel with fork...join. It samples the bus when PSEL/PENABLE and the matching PWRITE value are seen, waits for PREADY (raising a UVM_ERROR after 10 cycles without it), and publishes a transaction when the address or data changes from the previous one.

## 6.4 Scoreboard

Written data and read-back data are pushed into two queues. On each clock, when both queues have data, the scoreboard pops one entry from each, compares them, increments a pass or fail counter and prints the result. The report_phase prints the totals.

# 7. Simulation Setup

| Item | Value |
| --- | --- |
| Simulator | Siemens Questa 2024.1 |
| Top module | apb_top |
| Test | apb_base_test |
| Compile order | apb_if.sv, design.sv, pkg.sv, top.sv |
| Script | run.do |

Run with:

vsim -do run.do

# 8. Results

## 8.1 Scoreboard Output

## 8.2 UVM Report Summary

## 8.3 Waveforms

Write transfers. PSEL rises before PENABLE, and PREADY pulses during the access phase: PSEL, then PENABLE, PREADY pulse, address incrementing.

Read transfers. PRDATA matches the value written to the same address

## 8.4 Verification Summary

| ID | Feature | Result |
| --- | --- | --- |
| F1 | Write all addresses | [Pass / Fail] |
| F2 | Read all addresses | [Pass / Fail] |
| F3 | Data integrity | [x of 32 matched] |
| F4 | Protocol timing | [Pass / Fail, from waveforms] |
| F5 | PREADY watchdog | [No errors / errors] |